# Intersection Mix Zone with Random Turns on vnap:r2-p17: Test Summary

Rerun with the CAM timer rephase on every ID change, against an eavesdropper that links by identifiers, CAM timing and position

| Item | Value |
| --- | --- |
| Date | 2026-10-04, from 19:18:04 UTC, about 5 minutes of traffic |
| Scenario | vnap-docker/scenarios/c-its-pki-mixzone-random.toml (vnapctl), mobility seed 928157366 |
| Station image | vnap:r2-p17: the CAM timer restarts at a random phase on every ID change |
| Eavesdropper | links by shared identifiers, CAM timing phase (25 ms tolerance, 20 s window) and position continuity (15 s window, 50 m plus speed times gap); replays of the same capture with timing only and position only |
| Security | certs-v3, C-ITS-PKI root and AA; 24 butterfly authorization tickets, 6 per vehicle |
| Health | vnapctl check: PASS (43 expectations); all chain checks OK; 0 undecodable frames |
| Previous run | TestSummaries/mixzone-random-test-2026-10-04.docx (vnap:r2-p16, position-only eavesdropper) |

# 1. Purpose

The first intersection mix zone with random turns run used vnap:r2-p16, whose CAM timer kept running through each ID change. The timing of each car's CAMs therefore carried over to its new identity, and matching on that timing linked every change. vnap:r2-p17 restarts the CAM timer at a random phase on each change, and the eavesdropper now also tries timing linkage. This rerun checks that timing no longer links identities, and what position continuity achieves against the same traffic.

# 2. Setup

| Station | Starts on | Start | Pseudonym pool |
| --- | --- | --- | --- |
| rsu | the crossing, 40.0 N -8.0 E | - | regular AT |
| obu1 (id 2) | east road | 0 s | bke_at_0 .. 5 |
| obu2 (id 3) | north road | 1.5 s | bke_at_6 .. 11 |
| obu3 (id 4) | west road | 3 s | bke_at_12 .. 17 |
| obu4 (id 5) | south road | 4.5 s | bke_at_18 .. 23 |

Roads: four 150 m roads end on a square ring road. Each 600 m lap a car comes in on one road, turns left, straight or right at random (no U-turns), goes a random way along the ring and comes back in on the next road. Laps are 600 m at 36 km/h (60 s); the cars reach the crossing 1.5 s apart.

Mix zone and ID change: a pseudonym change event when a car enters the 40 m zone, then a full ETSI ID change and a silent period of 6-12 s (measured 6.1-11.4 s, mean 9.0 s).

CAM timer: 20 rephases logged, one per ID change ([IDCHANGE] CAM timer: new phase).

Ground truth: certificates from the stations' [PSEUDONYM] log lines; turns from the mobility client log.

## 2.1 Turns driven (ground truth)

| Lap | obu1 | obu2 | obu3 | obu4 |
| --- | --- | --- | --- | --- |
| 1 | east -> straight -> west | north -> right -> west | west -> right -> south | south -> left -> west |
| 2 | north -> straight -> south | south -> right -> east | west -> straight -> east | south -> left -> west |
| 3 | west -> left -> north | north -> straight -> south | north -> right -> west | south -> left -> west |
| 4 | west -> left -> north | west -> right -> south | north -> straight -> south | south -> right -> east |
| 5 | east -> straight -> west | east -> straight -> west | west -> straight -> east | south -> straight -> north |

Over the 20 crossings: 5 left, 9 straight, 6 right turns.

# 3. Results

| Measure | Value |
| --- | --- |
| Zone entries = ID changes | 20 (5 per car); every change was to a fresh identity |
| Links by the eavesdropper (timing and position) | 12: 0 correct, 12 wrong |
| of which by timing | 4: 0 correct |
| of which by position | 8: 0 correct |
| Replay, timing only | 5 links: 1 correct |
| Replay, position only | 13 links: 0 correct |
| Timing forced to choose for every change | 4 of 20 correct (20 %; guessing among 4 cars: about 25 %) |

## 3.1 Every link, scored against ground truth

| Time (UTC) | Track | Old identity | New identity | Technique | Evidence | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| 19:18:23 | V1 | obu3 | obu2 | position | 3.0 m, 6.97 s gap | WRONG |
| 19:18:24 | V5 | obu1 | obu3 | timing | 16 ms off after 11 CAMs | WRONG |
| 19:19:22 | V9 | obu4 | obu2 | position | 48.0 m, 3.59 s gap | WRONG |
| 19:19:25 | V5 | obu3 | obu1 | timing | 16 ms off after 9 CAMs | WRONG |
| 19:19:25 | V1 | obu2 | obu4 | timing | 12 ms off after 11 CAMs | WRONG |
| 19:20:23 | V5 | obu1 | obu3 | timing | 24 ms off after 10 CAMs; 22.0 m, 9.98 s gap | WRONG |
| 19:20:26 | V1 | obu4 | obu2 | position | 16.0 m, 8.42 s gap | WRONG |
| 19:21:24 | V5 | obu3 | obu1 | position | 18.0 m, 8.15 s gap | WRONG |
| 19:21:26 | V17 | obu4 | obu3 | position | 7.0 m, 7.66 s gap | WRONG |
| 19:22:24 | V17 | obu3 | obu1 | position | 24.0 m, 7.57 s gap | WRONG |
| 19:22:27 | V5 | obu1 | obu4 | position | 64.3 m, 13.72 s gap | WRONG |
| 19:22:28 | V20 | obu2 | obu3 | position | 21.0 m, 13.05 s gap | WRONG |

## 3.2 Timing and the pattern of position mistakes

Timing: with the CAM timer rephased, a new identity's first CAM bears no relation to the old identity's timing. The eavesdropper still makes a timing link when one silent car happens to predict that CAM within 25 ms and no other comes within 50 ms. Here that happened 5 times in the timing-only replay, 1 of them correct. Forced to choose for every change, timing got 4 of 20, which is what guessing gives.

Position-only replay, old car -> new car:

| Old identity belonged to | New identity belonged to | Links |
| --- | --- | --- |
| obu3 | obu1 | 3 |
| obu4 | obu2 | 2 |
| obu4 | obu3 | 2 |
| obu1 | obu3 | 1 |
| obu1 | obu4 | 1 |
| obu2 | obu1 | 1 |
| obu2 | obu3 | 1 |
| obu3 | obu2 | 1 |
| obu3 | obu4 | 1 |

The 13 wrong position links are spread over 9 pairings and change with the turns from lap to lap; no rule about the traffic turns them into correct links.

## 3.3 What the eavesdropper's tracks contain

| Track | Identities | Cars behind them, in order |
| --- | --- | --- |
| V1 | 4 | obu3 -> obu2 -> obu4 -> obu2 |
| V3 | 1 | obu2 |
| V4 | 1 | obu4 |
| V5 | 6 | obu1 -> obu3 -> obu1 -> obu3 -> obu1 -> obu4 |
| V8 | 1 | obu1 |
| V9 | 2 | obu4 -> obu2 |
| V11 | 1 | obu3 |
| V15 | 1 | obu1 |
| V17 | 3 | obu4 -> obu3 -> obu1 |
| V20 | 2 | obu2 -> obu3 |
| V21 | 1 | obu4 |
| V24 | 1 | obu2 |

# 4. Conclusion

| Run | Links | Correct | Reversible by a pattern? |
| --- | --- | --- | --- |
| Intersection Mix Zone with Random Turns, vnap:r2-p16, position-only eavesdropper (mixzone-random-test-2026-10-04.docx) | 18 | 1 | no (random turns) |
| vnap:r2-p16, timing-only replay (random turns, 3.5 min) | 16 | 16 | - |
| Intersection Mix Zone with Random Turns, vnap:r2-p17, timing and position (this run) | 12 | 0 | no |

With vnap:r2-p17 the eavesdropper linked no change correctly (12 links, all wrong), and neither timing nor position helps: timing is at chance level after the rephase, and with random turns the position mistakes have no pattern to undo. Against the techniques the eavesdropper has today, the random-turn mix zone hides which car is which.

# 5. Limitations of this run

One run, 5 minutes, 4 vehicles, one seed; the counts are not statistics.

Synchronized traffic: every car reaches the crossing every 60 s, 1.5 s apart.

Idealized positions (no GPS noise), equal speeds and identical vehicle attributes in the CAMs.

The eavesdropper decides each new identity greedily; it does not use heading, turn statistics, multiple hypotheses or vehicle attributes.

Only the CAM timer is rephased; GeoNetworking beacon timing and other periodic messages were not examined (only CAMs were sent in this scenario).

Safety cost: each car is silent about 9 s per lap, all of it inside the crossing.

# 6. Recommendations for subsequent scenarios

A turn-aware, multiple-hypothesis attacker. Use heading, learned turn probabilities and a joint assignment of all changes at a crossing, and report the attacker's uncertainty (anonymity set or entropy) rather than right and wrong links. Run it against both mix zone scenarios.

Other periodic timing. Check GeoNetworking beacons and other periodic messages (DENM, CPM, VAM when enabled) for the same timing carry-over, and add them to the eavesdropper's timing evidence.

Uneven turn probabilities and arrivals. Weighted turns (mostly straight on) and random arrival gaps, varying how many cars share the zone at once.

Repetition and scoring. At least 10 runs per configuration with different seeds, scored automatically against ground truth in vnapctl.

Realism. GPS noise, lane offsets, varying speeds and different vehicle dimensions.