# Intersection Mix Zone on vnap:r2-p17: Test Summary

Rerun with the CAM timer rephase on every ID change, against an eavesdropper that links by identifiers, CAM timing and position

| Item | Value |
| --- | --- |
| Date | 2026-10-04, from 19:17:41 UTC, about 5 minutes of traffic |
| Scenario | vnap-docker/scenarios/c-its-pki-mixzone.toml (vnapctl) |
| Station image | vnap:r2-p17: the CAM timer restarts at a random phase on every ID change |
| Eavesdropper | links by shared identifiers, CAM timing phase (25 ms tolerance, 20 s window) and position continuity (15 s window, 50 m plus speed times gap); replays of the same capture with timing only and position only |
| Security | certs-v3, C-ITS-PKI root and AA; 24 butterfly authorization tickets, 6 per vehicle |
| Health | vnapctl check: PASS (43 expectations); all chain checks OK; 0 undecodable frames |
| Previous run | TestSummaries/mixzone-test-2026-10-04.docx (vnap:r2-p16, position-only eavesdropper) |

# 1. Purpose

The first intersection mix zone run used vnap:r2-p16, whose CAM timer kept running through each ID change. The timing of each car's CAMs therefore carried over to its new identity, and matching on that timing linked every change. vnap:r2-p17 restarts the CAM timer at a random phase on each change, and the eavesdropper now also tries timing linkage. This rerun checks that timing no longer links identities, and what position continuity achieves against the same traffic.

# 2. Setup

| Station | Block (clockwise loop) | Arrives from | Leaves towards | Start | Pseudonym pool |
| --- | --- | --- | --- | --- | --- |
| rsu | at the crossing, 40.0 N -8.0 E | - | - | - | regular AT |
| obu1 (id 2) | north-east | east | north | 0 s | bke_at_0 .. 5 |
| obu2 (id 3) | north-west | north | west | 1.5 s | bke_at_6 .. 11 |
| obu3 (id 4) | south-west | west | south | 3 s | bke_at_12 .. 17 |
| obu4 (id 5) | south-east | south | east | 4.5 s | bke_at_18 .. 23 |

Roads: each car loops one 150 m block clockwise and turns right at the crossing, so it always arrives on one road and leaves on the next one clockwise. Laps are 600 m at 36 km/h (60 s); the cars reach the crossing 1.5 s apart.

Mix zone and ID change: a pseudonym change event when a car enters the 40 m zone, then a full ETSI ID change and a silent period of 6-12 s (measured 6.2-11.8 s, mean 8.7 s).

CAM timer: 20 rephases logged, one per ID change ([IDCHANGE] CAM timer: new phase).

Ground truth: certificates from the stations' [PSEUDONYM] log lines.

# 3. Results

| Measure | Value |
| --- | --- |
| Zone entries = ID changes | 20 (5 per car); every change was to a fresh identity |
| Links by the eavesdropper (timing and position) | 18: 0 correct, 18 wrong |
| of which by timing | 1: 0 correct |
| of which by position | 17: 0 correct |
| Replay, timing only | 2 links: 1 correct |
| Replay, position only | 19 links: 0 correct |
| Timing forced to choose for every change | 7 of 20 correct (35 %; guessing among 4 cars: about 25 %) |

## 3.1 Every link, scored against ground truth

| Time (UTC) | Track | Old identity | New identity | Technique | Evidence | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| 19:18:00 | V4 | obu3 | obu2 | position | 11.0 m, 6.93 s gap | WRONG |
| 19:18:02 | V3 | obu4 | obu3 | position | 3.0 m, 7.89 s gap | WRONG |
| 19:18:02 | V1 | obu2 | obu1 | position | 37.0 m, 10.4 s gap | WRONG |
| 19:18:57 | V9 | obu4 | obu1 | timing | 0 ms off after 3 CAMs | WRONG |
| 19:18:59 | V3 | obu3 | obu2 | position | 15.1 m, 5.28 s gap | WRONG |
| 19:19:02 | V1 | obu1 | obu3 | position | 63.8 m, 11.91 s gap | WRONG |
| 19:19:04 | V4 | obu2 | obu4 | position | 65.2 m, 12.1 s gap | WRONG |
| 19:20:01 | V1 | obu3 | obu2 | position | 4.7 m, 7.29 s gap | WRONG |
| 19:20:02 | V3 | obu2 | obu1 | position | 28.7 m, 9.95 s gap | WRONG |
| 19:20:02 | V4 | obu4 | obu3 | position | 2.7 m, 7.13 s gap | WRONG |
| 19:20:59 | V1 | obu2 | obu1 | position | 1.4 m, 7.12 s gap | WRONG |
| 19:21:03 | V3 | obu1 | obu4 | position | 13.4 m, 12.82 s gap | WRONG |
| 19:21:04 | V4 | obu3 | obu2 | position | 34.6 m, 11.24 s gap | WRONG |
| 19:21:05 | V17 | obu4 | obu3 | position | 32.6 m, 10.04 s gap | WRONG |
| 19:21:59 | V4 | obu2 | obu1 | position | 1.5 m, 7.96 s gap | WRONG |
| 19:22:01 | V3 | obu4 | obu3 | position | 11.4 m, 6.33 s gap | WRONG |
| 19:22:02 | V17 | obu3 | obu2 | position | 14.5 m, 9.14 s gap | WRONG |
| 19:22:05 | V1 | obu1 | obu4 | position | 8.5 m, 14.77 s gap | WRONG |

## 3.2 Timing and the pattern of position mistakes

Timing: with the CAM timer rephased, a new identity's first CAM bears no relation to the old identity's timing. The eavesdropper still makes a timing link when one silent car happens to predict that CAM within 25 ms and no other comes within 50 ms. Here that happened 2 times in the timing-only replay, 1 of them correct. Forced to choose for every change, timing got 7 of 20, which is what guessing gives.

Position-only replay, old car -> new car:

| Old identity belonged to | New identity belonged to | Links |
| --- | --- | --- |
| obu2 | obu1 | 5 |
| obu3 | obu2 | 5 |
| obu4 | obu3 | 5 |
| obu1 | obu4 | 4 |

19 of the 19 position links fall into the same four pairings as in the vnap:r2-p16 run: a car leaving on a road is matched with the car that arrived on that road, one road clockwise. The rotation does not depend on timing, so the CAM timer fix does not change it: an attacker who knows that every car turns right can still undo it and recover every car.

## 3.3 What the eavesdropper's tracks contain

| Track | Identities | Cars behind them, in order |
| --- | --- | --- |
| V1 | 6 | obu2 -> obu1 -> obu3 -> obu2 -> obu1 -> obu4 |
| V2 | 1 | obu1 |
| V3 | 6 | obu4 -> obu3 -> obu2 -> obu1 -> obu4 -> obu3 |
| V4 | 6 | obu3 -> obu2 -> obu4 -> obu3 -> obu2 -> obu1 |
| V9 | 2 | obu4 -> obu1 |
| V17 | 3 | obu4 -> obu3 -> obu2 |

# 4. Conclusion

| Run | Links | Correct | Reversible by a pattern? |
| --- | --- | --- | --- |
| Intersection Mix Zone, vnap:r2-p16, position-only eavesdropper (mixzone-test-2026-10-04.docx) | 17 | 2 | yes: one rotation |
| vnap:r2-p16, timing-only replay (random turns, 3.5 min) | 16 | 16 | - |
| Intersection Mix Zone, vnap:r2-p17, timing and position (this run) | 18 | 0 | yes: 19 of 19 position links in one rotation |

The timing leak is closed: timing links are rare coincidences. But the fixed-turn mix zone still fails against a pattern-aware attacker: every position link was wrong in the same predictable way (19 of 19 in one rotation), which an attacker can reverse. The CAM timer fix is necessary but not sufficient; the random-turn scenario is the one that holds.

# 5. Limitations of this run

One run, 5 minutes, 4 vehicles; the counts are not statistics.

Synchronized traffic: every car reaches the crossing every 60 s, 1.5 s apart.

Idealized positions (no GPS noise), equal speeds and identical vehicle attributes in the CAMs.

The eavesdropper decides each new identity greedily; it does not use heading, turn statistics, multiple hypotheses or vehicle attributes.

Only the CAM timer is rephased; GeoNetworking beacon timing and other periodic messages were not examined (only CAMs were sent in this scenario).

Safety cost: each car is silent about 9 s per lap, all of it inside the crossing.

# 6. Recommendations for subsequent scenarios

A turn-aware, multiple-hypothesis attacker. Use heading, learned turn probabilities and a joint assignment of all changes at a crossing, and report the attacker's uncertainty (anonymity set or entropy) rather than right and wrong links. Run it against both mix zone scenarios; on this scenario it should recover every car.

Retire fixed-turn mix zones as a privacy measure. Keep this scenario as the negative control for pattern-aware attacks; use random turns (c-its-pki-mixzone-random) for positive results.

Other periodic timing. Check GeoNetworking beacons and other periodic messages (DENM, CPM, VAM when enabled) for the same timing carry-over, and add them to the eavesdropper's timing evidence.

Uneven turn probabilities and arrivals. Weighted turns (mostly straight on) and random arrival gaps, varying how many cars share the zone at once.

Repetition and scoring. At least 10 runs per configuration with different seeds, scored automatically against ground truth in vnapctl.

Realism. GPS noise, lane offsets, varying speeds and different vehicle dimensions.