# Intersection Mix Zone Test Summary

Pseudonym unlinkability of four vehicles changing identity at a crossing, observed by a patient eavesdropper

| Item | Value |
| --- | --- |
| Date | 2026-10-04, 15:20:12 to about 15:25:20 UTC (about 5 minutes of traffic) |
| Scenario | vnap-docker/scenarios/c-its-pki-mixzone.toml (vnapctl, instance 1) |
| Station image | vnap:r2-p16; mobility client with mix zone triggers (vnap-docker/mobility) |
| Security | certs-v3 (TS 103 097 v2.1.1 / IEEE 1609.2), C-ITS-PKI root and AA; 24 butterfly authorization tickets, 6 per vehicle |
| Health | vnapctl check: PASS (43 expectations); all chain checks OK; 0 undecodable frames |
| Previous test | TestSummaries/convoy-test-2026-10-04.docx (three cars driving together) |

Note added 2026-10-04 (CAM timing leak): These results hold only against an eavesdropper that ignores CAM timing. In the image used for this run (vnap:r2-p16) the CAM timer kept running through each ID change, so every car's position within the 1 s CAM cycle carried over to its new identity. Matching on that timing alone linked 16 of 16 changes in the random-turn mix zone. vnap:r2-p17 restarts the CAM timer at a random phase on every ID change (the same matching then linked 1 of 13). Rerun the scenario with vnap:r2-p17 or later before relying on these numbers.

# 1. Purpose

The convoy test showed that cars changing identity together while close to each other confuse a patient eavesdropper (11 of 16 links wrong). It recommended an intersection mix zone as the next scenario: identity changes triggered by location, where vehicles arrive on one road and leave on another, with the silent period covering the crossing (ETSI TR 103 415 clauses 4.1.4-4.1.6). This test implements that scenario and measures it.

# 2. Setup

| Station | Block (clockwise loop) | Arrives from | Leaves towards | Start | Pseudonym pool |
| --- | --- | --- | --- | --- | --- |
| rsu | at the crossing, 40.0 N -8.0 E | - | - | - | regular AT |
| obu1 (id 2) | north-east | east | north | 0 s | bke_at_0 .. 5 |
| obu2 (id 3) | north-west | north | west | 1.5 s | bke_at_6 .. 11 |
| obu3 (id 4) | south-west | west | south | 3 s | bke_at_12 .. 17 |
| obu4 (id 5) | south-east | south | east | 4.5 s | bke_at_18 .. 23 |

Roads: each OBU drives around a 150 m x 150 m block at 36 km/h (10 m/s, 60 s per lap) and turns right at the crossing. Every road carries one car into the crossing and a different car out of it. The cars reach the zone 1.5 s apart (15 m).

Mix zone: a 40 m circle around the crossing. The mobility client, which knows the true positions, sends a pseudonym change event to each car as it enters (once per lap); there are no clock-driven changes.

ID change: full ETSI ID change (TS 102 723-8/-9: certificate, MAC, GeoNetworking address, stationId), then a silent period drawn per change from 6-12 s (measured: 6.1-11.9 s, mean 8.7 s). At 10 m/s the car drives 60-120 m while silent, from 40 m before the centre to 20-80 m along its exit road.

Eavesdropper: passive, on the V2X network only. It links a new identity to the nearest track that went silent within the last 15 s, if the distance is within 50 m plus speed times gap. It does not use direction.

Ground truth: certificates from the stations' own [PSEUDONYM] log lines; zone entries from the mobility client log.

# 3. Results

| Measure | Value |
| --- | --- |
| Zone entries (pseudonym change events) | 24 (6 per car, every 60 s) |
| Changes to a fresh identity | 20 (5 per car; each car's 6th change wrapped around to its first certificate) |
| Fresh identities seen by the eavesdropper | 19 (the last one was still silent when the log was taken) |
| Links made by the eavesdropper | 17 |
| Correct links (same car) | 2 (12 %) |
| Wrong links (two different cars joined) | 15 (88 %) |
| Observed fresh identities never linked | 2 |

## 3.1 Every link, scored against ground truth

| Time (UTC) | Track | Old identity | New identity | Distance | Gap | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| 15:20:21 | V1 | obu2 | obu1 | 9.0 m | 8.4 s | WRONG |
| 15:20:23 | V2 | obu4 | obu4 | 50.8 m | 7.0 s | correct |
| 15:20:24 | V3 | obu3 | obu2 | 21.0 m | 9.4 s | WRONG |
| 15:21:20 | V3 | obu2 | obu1 | 9.1 m | 6.6 s | WRONG |
| 15:21:21 | V8 | obu3 | obu2 | 9.1 m | 6.4 s | WRONG |
| 15:21:24 | V1 | obu1 | obu4 | 13.1 m | 12.0 s | WRONG |
| 15:21:28 | V2 | obu4 | obu3 | 36.8 m | 12.0 s | WRONG |
| 15:22:20 | V8 | obu2 | obu1 | 7.3 m | 6.6 s | WRONG |
| 15:22:24 | V2 | obu3 | obu2 | 22.7 m | 9.3 s | WRONG |
| 15:22:27 | V1 | obu4 | obu4 | 78.6 m | 11.0 s | correct |
| 15:23:21 | V16 | obu3 | obu2 | 5.4 m | 6.3 s | WRONG |
| 15:23:23 | V2 | obu2 | obu1 | 24.6 m | 9.7 s | WRONG |
| 15:23:24 | V1 | obu4 | obu3 | 0.6 m | 8.0 s | WRONG |
| 15:24:19 | V16 | obu2 | obu1 | 15.5 m | 5.7 s | WRONG |
| 15:24:24 | V20 | obu4 | obu3 | 0.5 m | 8.0 s | WRONG |
| 15:24:25 | V2 | obu1 | obu4 | 0.5 m | 13.0 s | WRONG |
| 15:24:25 | V1 | obu3 | obu2 | 34.5 m | 10.3 s | WRONG |

## 3.2 The wrong links follow one pattern

Old car -> new car, counted over all links:

| Old identity belonged to | New identity belonged to | Links | Road where it happens |
| --- | --- | --- | --- |
| obu2 | obu1 | 5 | north road: obu2 arrives, obu1 leaves |
| obu3 | obu2 | 5 | west road: obu3 arrives, obu2 leaves |
| obu4 | obu3 | 3 | south road: obu4 arrives, obu3 leaves |
| obu4 | obu4 | 2 | same car |
| obu1 | obu4 | 2 | east road: obu1 arrives, obu4 leaves |

A car that reappears on its exit road is closest to the spot where the car arriving on that road went silent, 40 m before the centre. The eavesdropper therefore links it to that car. Because every car turns right, this mistake is the same rotation every lap: obu1 is always mistaken for obu2, obu2 for obu3, and so on.

## 3.3 What the eavesdropper's tracks contain

| Track | Identities | Cars behind them, in order |
| --- | --- | --- |
| V1 | 6 | obu2 -> obu1 -> obu4 -> obu4 -> obu3 -> obu2 |
| V2 | 6 | obu4 -> obu4 -> obu3 -> obu2 -> obu1 -> obu4 |
| V3 | 3 | obu3 -> obu2 -> obu1 |
| V8 | 3 | obu3 -> obu2 -> obu1 |
| V16 | 3 | obu3 -> obu2 -> obu1 |
| V20 | 2 | obu4 -> obu3 |

# 4. Conclusion

Against the greedy position-continuity linker, the mix zone is the strongest result so far: 15 of 17 links (88 %) joined different cars, against 11 of 16 for the convoy and none for single vehicles or vehicles on separate roads. The only correct links were two of obu4's changes; obu4 is the last car to reach the crossing on each lap.

The protection is weaker than the error rate suggests, because the errors are deterministic. Every wrong link shifts identities one road clockwise, so an attacker who has learned the traffic pattern (all cars turn right here) can apply the inverse rotation and recover almost every car. Unlinkability needs confusion the attacker cannot predict, not just confusion.

# 5. Limitations of this run

One run, 5 minutes, 4 vehicles; the percentages are not statistics.

Fixed traffic pattern: same speed, same blocks and right turns on every lap; the arrival order never changes.

Idealized positions (no GPS noise) and identical vehicle attributes in the CAMs.

Simple attacker: greedy nearest fit, no use of heading, turn statistics, CAM attributes or multiple hypotheses.

Safety cost at the worst place: each car is silent for about 9 s per 60 s lap (about 14 % of the time), but all of it inside the crossing, where intersection collision warnings depend on CAMs. A real deployment would need to weigh that.

# 6. Recommendations for subsequent scenarios

Random turns. Let each car go straight, left or right at random on each pass (branching routes in the mobility client). This removes the fixed rotation, so the attacker's mistakes can no longer be reversed. This is the most important next step.

A turn-aware attacker. Give the eavesdropper a mode that uses heading and learned turn probabilities, and rerun this scenario. If it recovers the cars here, this confirms the weakness above; with random turns it should not.

Unequal arrivals. Random speeds and arrival gaps, and sometimes only one or two cars in the zone. Measure how linkability depends on the number of cars in the zone at the same time (the anonymity set).

Zone size and silence sweep. Radius 20-80 m against silent periods of 3-15 s; record the error rate and the CAM availability inside the crossing, to map the privacy against safety trade-off.

Several zones. A grid with two or more crossings, so the attacker must follow cars through several mix zones in sequence; errors should accumulate.

Realism. GPS noise, lane offsets and different vehicle dimensions, as recommended in the convoy summary.

Scoring and repetition. Automate ground-truth scoring in vnapctl and run each configuration at least 10 times.