# Convoy Scenario Test Summary

Pseudonym unlinkability of three vehicles driving together, observed by a patient eavesdropper

| Item | Value |
| --- | --- |
| Date | 2026-10-04, 15:06:48 to about 15:09:20 UTC (about 150 s of traffic) |
| Scenario | vnap-docker/scenarios/c-its-pki-convoy.toml (vnapctl, instance 1) |
| Station image | vnap:r2-p16 (Vanetza-NAP release2 with the vnap-secure patch set, including the position control channel) |
| Security | certs-v3 (TS 103 097 v2.1.1 / IEEE 1609.2), C-ITS-PKI root and AA; 24 butterfly authorization tickets, 8 per vehicle |
| Health | vnapctl check: PASS (31 expectations); all chain checks OK; 0 undecodable frames |

Note added 2026-10-04 (CAM timing leak): These results hold only against an eavesdropper that ignores CAM timing. In the image used for this run (vnap:r2-p16) the CAM timer kept running through each ID change, so every car's position within the 1 s CAM cycle carried over to its new identity. Matching on that timing alone linked 16 of 16 changes in the random-turn mix zone. vnap:r2-p17 restarts the CAM timer at a random phase on every ID change (the same matching then linked 1 of 13). Rerun the scenario with vnap:r2-p17 or later before relying on these numbers.

# 1. Purpose

Earlier runs showed that a full ETSI ID change (new certificate, MAC, GeoNetworking address and stationId) followed by a random 3-13 s silent period hides a vehicle from an eavesdropper that waits only 3 s, but not from one that waits 15 s: a single vehicle, or two vehicles on crossing roads, were re-linked after every change by position continuity. This test asks whether vehicles driving close together, changing identities at the same time, defeat that patient eavesdropper.

# 2. Setup

| Station | Role | Route and motion | Pseudonym pool |
| --- | --- | --- | --- |
| rsu | RSU, station 1 | static at 40.0, -8.0 | regular AT (no changes) |
| obu1 | OBU, station 2 | starts at t = 4 s | bke_at_0 .. 7 |
| obu2 | OBU, station 3 | starts at t = 6 s | bke_at_8 .. 15 |
| obu3 | OBU, station 4 | starts at t = 8 s | bke_at_16 .. 23 |

Movement: all three OBUs drive the same straight east-west road, 40.0 N from -8.003 to -7.997 (about 510 m, through the RSU), back and forth at a constant 50 km/h (13.9 m/s). Starting 2 s apart puts them about 28 m apart. The mobility client publishes each position 5 times per second on the control channel.

ID changes: the control client sends a pseudonym change event to every OBU every 20 s, so the three cars change at about the same moment. Each change is a full ETSI ID change (TS 102 723-8/-9), followed by a silent period drawn independently per car from 3-13 s (TR 103 415 4.1.4).

Eavesdropper: passive listener on the V2X network only (raw frames, no keys, no control channel). It decodes GeoNetworking and CAMs and links a new identity to a track that went silent within the last 15 s (link_window = 15) if the new position fits the old one plus speed times gap (link_distance 50 m).

Ground truth: the certificate each OBU used, from the stations' own [PSEUDONYM] log lines, which the eavesdropper cannot see.

# 3. Results

| Measure | Value |
| --- | --- |
| Identity changes per car | 8 (the 8th returned to the car's first certificate; see 5. Limitations) |
| Changes to a fresh identity | 21 (7 per car) |
| Links made by the eavesdropper | 16 |
| Correct links (same car) | 5 (31 %) |
| Wrong links (two different cars joined) | 11 (69 %) |
| Fresh identities never linked | 5 |
| OBU tracks at the end | 8 (for 3 real vehicles) |

## 3.1 Every link, scored against ground truth

| Time (UTC) | Track | Old identity | New identity | Distance | Gap | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| 15:07:12 | V2 | obu1 | obu1 | 97.3 m | 7.0 s | correct |
| 15:07:14 | V1 | obu2 | obu2 | 125.0 m | 9.0 s | correct |
| 15:07:30 | V2 | obu1 | obu1 | 14.0 m | 5.0 s | correct |
| 15:07:36 | V7 | obu3 | obu2 | 16.7 m | 10.5 s | WRONG |
| 15:07:38 | V1 | obu2 | obu3 | 44.5 m | 13.5 s | WRONG |
| 15:07:50 | V2 | obu1 | obu1 | 69.5 m | 5.0 s | correct |
| 15:07:53 | V7 | obu2 | obu2 | 111.2 m | 8.0 s | correct |
| 15:08:09 | V2 | obu1 | obu3 | 5.6 m | 4.3 s | WRONG |
| 15:08:16 | V12 | obu3 | obu2 | 105.7 m | 10.5 s | WRONG |
| 15:08:31 | V16 | obu1 | obu2 | 52.8 m | 5.8 s | WRONG |
| 15:08:33 | V12 | obu2 | obu3 | 91.7 m | 8.5 s | WRONG |
| 15:08:53 | V19 | obu1 | obu3 | 61.1 m | 8.4 s | WRONG |
| 15:08:55 | V16 | obu2 | obu1 | 169.4 m | 10.2 s | WRONG |
| 15:08:57 | V12 | obu3 | obu2 | 186.1 m | 11.5 s | WRONG |
| 15:09:11 | V16 | obu1 | obu2 | 52.8 m | 5.8 s | WRONG |
| 15:09:13 | V12 | obu2 | obu3 | 77.8 m | 7.6 s | WRONG |

## 3.2 What the eavesdropper's tracks contain

Each row is one track at the end of the run; the cars are listed in the order its identities were seen.

| Track | Identities | Cars behind them, in order |
| --- | --- | --- |
| V1 | 3 | obu2 -> obu2 -> obu3 |
| V2 | 5 | obu1 -> obu1 -> obu1 -> obu1 -> obu3 |
| V3 | 1 | obu3 |
| V7 | 3 | obu3 -> obu2 -> obu2 |
| V12 | 5 | obu3 -> obu2 -> obu3 -> obu2 -> obu3 |
| V16 | 4 | obu1 -> obu2 -> obu1 -> obu2 |
| V19 | 2 | obu1 -> obu3 |
| V25 | 1 | obu1 |

## 3.3 Observations

The first 40 s were tracked correctly. The first three links (15:07:12-15:07:30) were right: before the random silent periods had shuffled the cars, each reappeared where only it could be.

After that, links were mostly wrong. From 15:07:36 on, 11 of 13 links joined two different cars. When the cars are about 28 m apart and each is silent for a different random time, the first car to reappear near a lost track's predicted position is often another car. Wrong links were made at 5.6-186 m and 4.3-13.5 s gaps, inside the eavesdropper's limits.

Only one track stayed with one car for long. V2 followed obu1 through three changes (about 40 s) before it jumped to obu3. Every other multi-identity track mixes two or three cars, so a route reconstructed from it belongs to no single vehicle.

The group is still visible. The eavesdropper knows that three cars drove this road together during the whole run. The convoy hides which car is which, not that the convoy was there.

Safety cost. While silent, a car sends no CAMs: with 3-13 s silence every 20 s, each car was off the air for an average of about 40 % of the time.

# 4. Conclusion

Driving close together with synchronized ID changes and independent random silent periods defeated position-continuity linking in this run: 11 of 16 links (69 %) were wrong, against 0 wrong links (and every change linked) for two cars on crossing roads and for a single car. The convoy acts as a moving mix zone. The protection depends on the other cars being close enough to be confused with, and it comes with a large loss of CAM availability.

# 5. Limitations of this run

One run of about 150 s; the percentages are not statistics. Repeated runs are needed.

Idealized motion: constant, identical speeds on one straight road, exact positions (1 m confidence, no GPS noise). Identical speeds make dead reckoning less useful to the eavesdropper (every car fits); real traffic has speed differences that can help or hurt it.

Identical vehicles: all OBUs send the same vehicle length, width and type in their CAMs. Real vehicles differ, which an eavesdropper can use as a fingerprint across ID changes.

Simple attacker: a greedy nearest-fit linker. A multiple-hypothesis or Kalman-filter tracker, or one that also uses CAM attributes, could do better.

Pool wrap-around: each car's 8th change returned to its first certificate (and so its first MAC, GN address and stationId). The eavesdropper re-identified those by identifier; they are not counted as links above, but reuse is a real linking risk.

# 6. Recommendations for subsequent scenarios

The scenarios below aim to make the eavesdropper's job harder in realistic ways, and to measure how robust that is against a stronger attacker. Each lists what the harness needs.

Intersection mix zone. Two or three streams of traffic on crossing roads, timed to meet at an intersection, with ID changes triggered while the vehicles are inside the zone and silent periods covering the crossing. Vehicles leave on different roads than they arrived on, so the attacker must also guess the turn. Needs: a location-triggered change mode in the control client (change when within r metres of a point), and routes with branches.

Speed variation and overtaking during silence. Give each vehicle a speed profile (random acceleration and braking, overtakes) so that the order of the cars changes while they are silent and a dead-reckoned prediction no longer points at the right car. Needs: speed profiles or random speed changes in the mobility client.

Denser traffic. Six to ten vehicles in mixed groups and directions, so that more candidates fall inside the attacker's prediction window. Needs: more butterfly ATs (pool size at least the changes per run, e.g. 8-10 per vehicle) and more station addresses.

Cooperative change instead of a fixed clock. Change identity only when at least k neighbours also change (for example an RSU or group leader sending IDCHANGE-TRIGGER to all vehicles in range), and compare with unsynchronized random changes (control client mode random) as a control. Shows how much of the effect comes from synchronization.

Silent period versus attacker window. Sweep the silent period (0, 3-13 s, 10-30 s) against eavesdropper link windows of 3, 15, 30 and 60 s, and record both linking accuracy and CAM availability at the RSU. This maps the privacy against safety trade-off instead of a single point.

Realistic positions. Add GPS noise (1-3 m, with a matching position confidence) and lane offsets in the mobility client; exact positions make continuity easier to judge.

Vehicle fingerprints. Give vehicles different lengths, widths and types, then repeat with harmonized values, to measure how much CAM attributes help the attacker across ID changes and whether they should be coarsened.

Group dispersal. Let the convoy split after a few changes (vehicles turn off onto different roads). A late split lets a patient attacker recover identities later; this tests how long confusion lasts.

Pseudonym reuse as its own scenario. Small pools that wrap around, scored against an attacker that matches recurring identifiers and identities that come and go together. Keeps reuse out of the other measurements.

Stronger attacker baselines. Add a multiple-hypothesis or Kalman-filter tracker, use of CAM attributes and path history, and several eavesdroppers along the road. A protection that only defeats the greedy linker is weak.

Scoring and repetition. Automate ground-truth scoring in vnapctl (correct and wrong links, track purity, time a car is followed correctly) and run each scenario at least 10 times with different seeds for statistics.