# Intersection Mix Zone with Random Turns: Test Summary

Pseudonym unlinkability of four vehicles that change identity at a crossing and turn at random, observed by a patient eavesdropper

| Item | Value |
| --- | --- |
| Date | 2026-10-04, 16:02:24 to about 16:07:40 UTC (about 5 minutes of traffic) |
| Scenario | vnap-docker/scenarios/c-its-pki-mixzone-random.toml (vnapctl, instance 1), mobility seed 675733204 |
| Station image | vnap:r2-p16; mobility client with random-turn laps and mix zone triggers (vnap-docker/mobility) |
| Security | certs-v3 (TS 103 097 v2.1.1 / IEEE 1609.2), C-ITS-PKI root and AA; 24 butterfly authorization tickets, 6 per vehicle |
| Health | vnapctl check: PASS (43 expectations); all chain checks OK; 0 undecodable frames |
| Previous tests | TestSummaries/mixzone-test-2026-10-04.docx (same crossing, fixed right turns); convoy-test-2026-10-04.docx |

# 1. Purpose

In the first mix zone test every car always turned right on the same block. The eavesdropper joined different cars in 15 of 17 links, but always in the same way: identities shifted one road clockwise, a rotation an attacker who knows the traffic pattern could undo. That summary recommended random turns as the next step. This test repeats the scenario with random turns and random return roads, to check whether the eavesdropper's mistakes stop being predictable.

# 2. Setup

| Station | Starts on | Start | Pseudonym pool |
| --- | --- | --- | --- |
| rsu | the crossing, 40.0 N -8.0 E | - | regular AT |
| obu1 (id 2) | east road | 0 s | bke_at_0 .. 5 |
| obu2 (id 3) | north road | 1.5 s | bke_at_6 .. 11 |
| obu3 (id 4) | west road | 3 s | bke_at_12 .. 17 |
| obu4 (id 5) | south road | 4.5 s | bke_at_18 .. 23 |

Roads: a four-way crossing with four 150 m roads (arms) that end on a square ring road. Each lap a car comes in on one arm, turns left, straight or right at random at the crossing (no U-turns), drives to the end of its exit arm, takes a random direction along the ring to the next arm and comes back in on it. That arm is the next lap's arrival road, so where a car comes from is random too.

Timing: every lap is 600 m at 36 km/h (60 s), so the four cars reach the crossing 1.5 s apart on every lap; all four are in or near the zone together.

Mix zone: a 40 m circle around the crossing; the mobility client sends each car a pseudonym change event as it enters, once per lap. There are no clock-driven changes.

ID change: full ETSI ID change (TS 102 723-8/-9), then a silent period drawn per change from 6-12 s (measured 6.0-11.7 s, mean 8.9 s), which covers the turn: a car reappears 20-80 m along its exit road.

Eavesdropper: passive, on the V2X network only. It links a new identity to the nearest track that went silent within the last 15 s, if the distance is within 50 m plus speed times gap. It does not use direction.

Ground truth: certificates from the stations' [PSEUDONYM] log lines; each lap's turns from the mobility client log.

## 2.1 Turns driven (ground truth)

| Lap | obu1 | obu2 | obu3 | obu4 |
| --- | --- | --- | --- | --- |
| 1 | east -> right -> north | north -> left -> east | west -> left -> north | south -> straight -> north |
| 2 | east -> right -> north | south -> left -> west | west -> straight -> east | west -> left -> north |
| 3 | east -> right -> north | south -> right -> east | south -> straight -> north | east -> left -> south |
| 4 | east -> left -> south | south -> right -> east | west -> straight -> east | west -> right -> south |
| 5 | west -> right -> south | north -> straight -> south | north -> straight -> south | east -> straight -> west |
| 6 | west -> straight -> east | west -> right -> south | east -> right -> north | south -> right -> east |

Over the 24 crossings: 6 left, 8 straight, 10 right turns. Each entry reads arrival road -> turn -> exit road.

# 3. Results

| Measure | Value |
| --- | --- |
| Zone entries (pseudonym change events) | 24 (6 per car, every 60 s) |
| Changes to a fresh identity | 20 (5 per car; each car's 6th change wrapped around to its first certificate) |
| Fresh identities seen by the eavesdropper | 20 |
| Links made by the eavesdropper | 18 |
| Correct links (same car) | 1 (6 %) |
| Wrong links (two different cars joined) | 17 (94 %) |
| Observed fresh identities never linked | 2 |

## 3.1 Every link, scored against ground truth

| Time (UTC) | Track | Old identity | New identity | Distance | Gap | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| 16:02:42 | V3 | obu1 | obu2 | 19.0 m | 9.8 s | WRONG |
| 16:02:42 | V2 | obu2 | obu1 | 1.0 m | 8.3 s | WRONG |
| 16:02:45 | V1 | obu3 | obu4 | 53.3 m | 8.7 s | WRONG |
| 16:02:48 | V4 | obu4 | obu3 | 117.0 m | 10.3 s | WRONG |
| 16:03:43 | V1 | obu4 | obu1 | 73.8 m | 5.8 s | WRONG |
| 16:03:46 | V4 | obu3 | obu2 | 25.0 m | 10.2 s | WRONG |
| 16:03:48 | V3 | obu2 | obu3 | 89.4 m | 13.8 s | WRONG |
| 16:04:41 | V13 | obu4 | obu1 | 52.4 m | 2.8 s | WRONG |
| 16:04:46 | V1 | obu1 | obu3 | 75.1 m | 13.5 s | WRONG |
| 16:04:50 | V3 | obu3 | obu4 | 33.0 m | 13.7 s | WRONG |
| 16:05:41 | V16 | obu2 | obu1 | 15.0 m | 6.3 s | WRONG |
| 16:05:43 | V13 | obu1 | obu3 | 22.0 m | 10.5 s | WRONG |
| 16:05:46 | V3 | obu4 | obu2 | 114.0 m | 8.6 s | WRONG |
| 16:05:49 | V1 | obu3 | obu4 | 81.8 m | 12.7 s | WRONG |
| 16:06:41 | V1 | obu4 | obu1 | 52.0 m | 2.8 s | WRONG |
| 16:06:43 | V16 | obu1 | obu2 | 65.7 m | 10.7 s | WRONG |
| 16:06:45 | V13 | obu3 | obu3 | 90.0 m | 9.0 s | correct |
| 16:06:47 | V3 | obu2 | obu4 | 66.5 m | 12.5 s | WRONG |

## 3.2 The wrong links no longer follow one pattern

Old car -> new car, counted over all links:

| Old identity belonged to | New identity belonged to | Links |
| --- | --- | --- |
| obu3 | obu4 | 3 |
| obu4 | obu1 | 3 |
| obu1 | obu2 | 2 |
| obu1 | obu3 | 2 |
| obu2 | obu1 | 2 |
| obu2 | obu3 | 1 |
| obu2 | obu4 | 1 |
| obu3 | obu2 | 1 |
| obu3 | obu3 | 1 |
| obu4 | obu2 | 1 |
| obu4 | obu3 | 1 |

In the fixed-turn run the 15 wrong links fell into 4 pairings, one per road, each repeated every lap. Here the 17 wrong links are spread over 10 different pairings, and which car is mistaken for which changes from lap to lap with the turns.

Undoing the mistakes now needs the turns. An attacker that assumes every car takes the same turn, the rule that recovers every car in the fixed-turn run, is right for at most 10 of the 24 crossings here (42 %, assuming 'right'), not far above the 33 % of guessing among three turns.

## 3.3 What the eavesdropper's tracks contain

| Track | Identities | Cars behind them, in order |
| --- | --- | --- |
| V1 | 6 | obu3 -> obu4 -> obu1 -> obu3 -> obu4 -> obu1 |
| V2 | 2 | obu2 -> obu1 |
| V3 | 6 | obu1 -> obu2 -> obu3 -> obu4 -> obu2 -> obu4 |
| V4 | 3 | obu4 -> obu3 -> obu2 |
| V13 | 4 | obu4 -> obu1 -> obu3 -> obu3 |
| V16 | 3 | obu2 -> obu1 -> obu2 |

# 4. Conclusion

| Scenario | Links | Wrong | Mistakes reversible by a pattern? |
| --- | --- | --- | --- |
| Single vehicle or two cars on separate roads (15 s window) | all changes | 0 | - |
| Convoy (convoy-test-2026-10-04) | 16 | 11 | partly (the group is visible) |
| Mix zone, fixed right turns (mixzone-test-2026-10-04) | 17 | 15 | yes: one rotation |
| Mix zone, random turns (this test) | 18 | 17 | no: best fixed rule 10/24 |

With random turns and random return roads, the greedy eavesdropper joined different cars in 17 of 18 links (94 %), and its mistakes no longer form a pattern that a rule about the traffic can reverse. Of the scenarios tested so far this is the first where the eavesdropper ends up neither right nor systematically wrong: after a few laps each of its tracks is a mix of cars it cannot untangle without knowing the turns.

This is still a weak attacker. A tracker that uses heading, keeps several hypotheses open and learns turn probabilities would do better than the greedy linker, but with three equally likely turns and four cars in the zone together it has to choose among several equally plausible assignments at every crossing.

# 5. Limitations of this run

One run, 5 minutes, 4 vehicles, one seed; the percentages are not statistics.

Synchronized traffic: every car reaches the crossing every 60 s, 1.5 s apart, so the zone always holds four cars. Real traffic is burstier; a car alone in the zone gains nothing.

Uniform turns. Real junctions have uneven turn probabilities (mostly straight on), which help an attacker.

Idealized positions (no GPS noise), equal speeds and identical vehicle attributes in the CAMs.

Simple attacker: greedy nearest fit without heading, turn statistics, CAM attributes or multiple hypotheses.

Safety cost: each car is silent about 9 s per 60 s lap (about 15 % of the time), all of it inside the crossing, where intersection collision warnings depend on CAMs.

# 6. Recommendations for subsequent scenarios

A stronger, turn-aware attacker. Add a mode to the eavesdropper that uses heading, keeps multiple hypotheses and learns turn probabilities, and rerun both mix zone scenarios. Report the attacker's uncertainty (anonymity set size or entropy per crossing), not only right and wrong links. This is now the most informative next step.

Uneven turn probabilities. Weight the turns (e.g. 60 % straight, 20 % left, 20 % right) to see how much realistic junction statistics help that attacker.

Anonymity set size. Vary how many cars share the zone at once (1, 2, 4, 8) with random arrival gaps, to find how many are needed before a change in the zone is worth the silence.

Several crossings. A grid of two or more mix zones, so the attacker must follow cars through several crossings; uncertainty should compound.

Zone size and silence sweep. Radius 20-80 m against silent periods of 3-15 s, recording linking and CAM availability in the crossing (the privacy against safety trade-off).

Realism. GPS noise, lane offsets, varying speeds and different vehicle dimensions.

Scoring and repetition. Automate ground-truth scoring in vnapctl and run each configuration with at least 10 seeds (control.mobility_seed) for statistics.