# VNAP-Secure: Overhead of Pseudonym Changes and the Privacy-Safety Trade-off

Design proposal for review · draft 0.2 · 2026-10-16 · scope: computation load on the OBU and on its neighbours, the disruption of communication that may affect safety, and a joint experiment that weighs both against privacy

# 1. Summary

Pseudonym changes protect privacy at a price: work for the vehicle and its neighbours, and moments in which the neighbours know less about the vehicle. The results so far measure the privacy side in detail. The cost side has only a few numbers (refill times, the share of time spent silent). This design adds measurements of the cost, so that every privacy result can be read against what it costs.

Computation: the cost of one change and of one refill, on the OBU and on the vehicles that receive from it, as times and as counts of cryptographic operations.

Disruption: what receivers lose around a change: the gap in reception, the error in where they believe the vehicle is, the time their information is stale, verification failures, and delayed or lost warnings.

The method is a comparison on the same seed: a run without changes as the baseline, and the same scenario with changes at increasing rates and with longer silent periods. The overhead is the difference; against the change rate it gives a cost per change.

A joint experiment (Section 9) then scores the same runs for privacy and for cost. It places each countermeasure on a chart of privacy against safety, with confidence intervals, and names the policies that no other tested policy beats on both. Its result is a comparison within the simulated conditions; Section 9.8 says what it cannot claim and what further evidence a recommendation to manufacturers would need.

The first phase needs no new station image. It computes the disruption measures from data that runs already save, plus a recording of the vehicles' true positions (Section 11).

# 2. Background

## 2.1 What a change does

| Step | Work | Effect on communication |
| --- | --- | --- |
| ID change notification (PREPARE, COMMIT) | routers locked; new GN address, MAC and `stationId`, each derived with SHA-256; switch to the next certificate and key | nothing is sent with the old identifiers once PREPARE has started |
| CAM timer restart | none worth counting | the first CAM after the change is skipped, the next comes after a random delay of up to one interval: up to two intervals without a CAM |
| Silent period (optional) | none | all outgoing frames are dropped for a random time (`silent_min_ms` to `silent_max_ms`): CAMs, beacons and injected messages, warnings included |
| First signed message | the full new certificate is attached | receivers must check the new certificate's chain once |
| Refill (with `[pki]`) | a request over the control channel; per certificate: private key derivation, key check, chain check | none, unless the pool runs empty: changes are then refused |

## 2.2 Already measured

| Measure | Where | Covers |
| --- | --- | --- |
| `<station>.pki.refresh_ms_mean`, `refresh_ms_max` | `vnapctl check` | time from a refill request to its installation |
| `<station>.pki.derivation_ms_mean` | `vnapctl check` | key derivation per batch |
| `<station>.pki.starved` | `vnapctl check` | changes refused for lack of certificates |
| `pki.issue_ms_max`, `pki.queue_ms_max` | `vnapctl check` | work and queueing at the PKI |
| `silent period over, <n> frame(s) suppressed` | station log | frames dropped per silent period |
| `silent.share` | summary report facts | share of the vehicles' time spent silent |
| `rsu.cam.rx_per_s` | `vnapctl check` | CAMs per second at the RSU, over the whole run |
| every frame on the link, with time, sender identifiers and position | eavesdropper `messages.jsonl` | raw material for the disruption measures (Section 6) |

None of these is aligned with individual changes, none measures CPU, and none measures what receivers believe about a vehicle during a gap.

# 3. Goals and non-goals

## 3.1 Goals

A cost per change and per refill, in CPU time and in cryptographic operations, for the OBU and for its neighbours.

Disruption measures per change and per run, at every receiver, that describe what a safety application would have lost.

A sweep that shows how both grow with the change rate and the silent period, next to the privacy result of the same runs.

A joint experiment, defined before it is run, that compares countermeasures on privacy and safety at once and states the result with its uncertainty (Section 9).

Measures available in `vnapctl check`, `score.json`-like run output and the summary report, so that scenarios can state limits (`expect`).

## 3.2 Non-goals

Absolute OBU figures. Containers on a VM are not OBU hardware. Times are for comparison; the operation counts are what carries over (Section 5.3).

Radio effects. The link is a lossless bridge, without loss, congestion or range. The disruption measured is the part the change causes, a lower bound (Section 12).

Judging safety applications. The measures describe what information was missing, not whether a particular application would have failed.

# 4. Method

Baseline: the scenario with pseudonym changes switched off, on the same mobility seed, so that vehicles drive the same paths.

Variants: the same scenario with changes every 60, 30, 10 and 5 s, each with no silent period, 3 to 13 s (the SAE J2735 values reported in ETSI TR 103 415) and 6 to 12 s.

Alignment: every measure is kept per change, with the change's time from the station log (`[IDCHANGE]` lines), not only as a run average. A run average hides a 10 s gap among minutes of normal traffic.

Cost per change: the slope of a measure against changes per minute, over the variants.

Seeds: 3 per variant to start. 13 variants (one baseline, 12 with changes) of 3 minutes each take about two hours per scenario.

Scenarios: `c-its-pki-traffic` (two vehicles on crossing roads) and `c-its-pki-mixzone-random` (four vehicles; changes synchronized in the mix zone, so refills arrive together).

# 5. Computation load

## 5.1 On the OBU

| Work | When | How to measure |
| --- | --- | --- |
| ID change handshake and switch | every change | a timing log line at COMMIT, e.g. `[IDCHANGE] completed in <us> us` (new, station code) |
| signing | every message; unchanged by a change | counter of signatures made (new) |
| key derivation and key check | every refill, per certificate | exists: `derivation_ms_mean`; add a count |
| chain check of new certificates | every refill, per certificate | counter and total time (new) |
| all of the above, and anything missed | the whole run | CPU time of the station's container, sampled from its cgroup about 10 times a second (new, harness) |

The cgroup samples are aligned with the change and refill times. The extra CPU in a window around each event, against the baseline, catches work the counters do not.

## 5.2 On the neighbours

Every receiver of a changed vehicle sees an unknown certificate and checks its chain once. With many vehicles changing, for example together in a mix zone, this work arrives at every receiver at the same moment.

counters at each receiver: certificate cache misses, chain checks and their time (new, station code);

the same cgroup samples at the RSU and the other OBUs, aligned with the changes of the vehicles they hear.

## 5.3 Carrying the results over to real hardware

CPU times on shared VM cores depend on the host and on the other containers. The design therefore reports both times and counts per change: signatures made and verified, chain checks, keys derived. Multiplied by the per-operation times of a target OBU or its hardware security module, the counts give an estimate for that hardware. The times serve to compare variants within one host.

# 6. Communication disruption

Disruption is measured at the receivers, because what matters for safety is what the neighbours know about the vehicle. Receivers are the RSU and every other OBU.

## 6.1 Measures

| Measure | Definition | Why it matters |
| --- | --- | --- |
| Reception gap | per change and receiver: time from the last message under the old identity to the first under the new one | for that long the vehicle is missing from the receiver's view |
| Position error | the receiver's belief about the vehicle's position against its true position, over time; two variants: last known position, and last position extended by speed and heading | turns a gap into metres of uncertainty, which is what a collision warning depends on |
| Time stale | share of time the receiver's latest information about the vehicle is older than a threshold (proposed 1 s, the longest CAM interval of EN 302 637-2) | one number per vehicle and run, comparable across variants |
| Verification failures | received messages whose security report is not `Success`, within 5 s after a change | a receiver that cannot yet check the new certificate may discard messages |
| Ghost time | time the old identity remains in the receiver's neighbour table while the new one is already there, under a stated table timeout | the receiver briefly counts two vehicles where there is one, and loses the vehicle's history |
| Warning delay and loss | delay from triggering a DENM to its reception, and the share never received (Section 6.3) | the most direct safety cost: a silent period also drops warnings |

The minimum cost of a change, without a silent period, is set by the CAM timer restart: up to two CAM intervals without a CAM. The measured gap should never be shorter; if it is longer, something else delays the first message.

## 6.2 Data sources

Reception gap, position belief, ghost time: the eavesdropper's `messages.jsonl` already holds every frame with time, identifiers and position. On the lossless link every receiver hears the same frames, so the eavesdropper serves as a reference receiver. The mapping from identity to vehicle comes from the station logs, as for scoring.

True positions: the mobility client publishes each vehicle's position 5 times a second on the control broker, but logs it only every 10 s. A recorder of those positions is needed (new, harness).

Verification results: each receiving station publishes received messages with their security report on `vanetza/out/cam` on its own broker. A collector records them during the run (new, harness).

## 6.3 Warning probe

A probe publishes DENMs on a vehicle's `vanetza/in/` topic at random times, about once every 10 s, and records when and whether each receiver gets them. A warning triggered during a silent period is dropped with the other frames, so the probe measures how often that happens and for how long warnings would be held back.

This also allows a defence to be measured: ending the silent period early, or exempting warnings from it, when a warning has to go out. That is a change to the station code and is left to a later decision (Section 13).

# 7. Outputs

Proposed `vnapctl check` metrics, so that a scenario can state limits:

| Metric (proposed) | Meaning |
| --- | --- |
| `<station>.change.ms_mean`, `.ms_max` | time of the change handshake on the station |
| `<station>.change.cpu_ms_mean` | extra CPU per change against the baseline window |
| `<station>.verify.chain_checks`, `.cache_misses` | work caused by other vehicles' changes |
| `<receiver>.gap_ms_max`, `.gap_ms_mean` | reception gaps around changes, over all senders |
| `<receiver>.stale_share` | share of time information about some sender was older than the threshold |
| `<receiver>.pos_error_m_p95` | 95th percentile of the position error during gaps |
| `denm.delay_ms_max`, `denm.lost` | warning probe results |

Per run: an `overhead.json` in the run's results, with every change and its measures, next to `score.json`.

Summary report: a section on cost in `report.docx`, and facts for `recommendations.toml` (for example a rule when warnings were lost).

Results tab: the per-run figures next to the privacy result.

# 8. Relation to the adversarial agents design

`VNAP-Secure-Adversarial-Agents-Design.docx` weighs a defence's privacy against `silent.share` only. The measures here give that loop a cost that is closer to safety: stale time, position error and lost warnings. They would replace or join `silent.share` in the blue agent's score, and lost warnings would become a hard constraint.

In the other direction, the joint experiment (Section 9) uses the strongest attacker in that design's archive, once there is one, so that a countermeasure is not judged only against the attacker that happens to exist today.

# 9. Joint experiment: privacy against safety

The measures above describe cost; the eavesdropper's scoring describes privacy. This section defines one experiment that produces both for the same runs, so that countermeasures can be compared on the two at once. Everything in it (factors, measures, hypotheses, analysis) is fixed before the runs, so that the result cannot be shaped afterwards by choosing the measure that looks best.

## 9.1 Question

For each pseudonym-change policy, under each traffic condition: how much privacy does it give, what does it cost in safety-relevant information, and which policies are not beaten on both by another tested policy?

## 9.2 Factors

| Factor | Levels | Notes |
| --- | --- | --- |
| Change trigger | periodic every 30 s; periodic every 10 s; mix zone, `min_vehicles` 1; mix zone, `min_vehicles` 2; dynamic mix zone | existing scenario keys |
| Silent period | none; 3 to 13 s; 6 to 12 s | `silent_min_ms`, `silent_max_ms` |
| Baseline | no pseudonym changes | the reference for every cost measure |
| Vehicles | 2, 4, 8 | 8 needs a new scenario on the crossing; to be confirmed that the host carries it |
| Speeds | all equal; spread (e.g. 30 to 50 km/h) | equal speeds favour the defender in a mix zone |
| Attacker | A0 with `link_window` 15 s, both techniques; later the strongest attacker in the adversarial archive; a guessing attacker | attackers are replayed on the same frames, so they cost no runs |

Held fixed: full ID change, the CAM timer restart (station image `vnap:r2-p17` or later), random turns at the crossing, certificate refill from the run's PKI, and the warning probe of Section 6.3. A policy is one combination of trigger and silent period: 15 policies and the baseline.

## 9.3 Primary and secondary measures

One primary measure per side is declared now. The others are reported, but a conclusion rests on the primary ones.

| Side | Primary | Secondary |
| --- | --- | --- |
| Privacy | tracking success: correct links / pseudonym changes made, for the strongest attacker | longest chain followed; correct links relative to the guessing attacker; mistake share |
| Safety | stale share at receivers: share of time information about a vehicle is older than 1 s | position error during gaps (95th percentile, extended variant); reception gap; warning delay |
| Hard limit | no lost warnings outside what the silent period itself explains | warning loss rate |
| Cost | reported, not optimised | chain checks per receiver and minute; CPU per change; certificates per vehicle and hour |

The guessing attacker links every new pseudonym to a random silent track. Replayed on the same frames, it gives the chance level of each run empirically, instead of the formula 1 / vehicles, which ignores how many vehicles were actually silent at the moment.

## 9.4 Hypotheses

| Id | Hypothesis | What it would show |
| --- | --- | --- |
| H1 | A silent period lowers tracking success to chance level only when several vehicles change together (mix zone with `min_vehicles` 2, dynamic mix zone); with periodic triggers it adds stale time without that effect | where silence is worth its cost |
| H2 | Stale share is close to changes per second × (mean silent period + two CAM intervals) | a check that the measurement works; a large deviation means something else delays the first message |
| H3 | Warning loss is close to the share of time spent silent | a check of the probe |
| H4 | A dynamic mix zone reaches the tracking success of a fixed mix zone at a lower stale share | whether the dynamic zone is the better policy |
| H5 | The policies on the frontier differ between 2, 4 and 8 vehicles | whether a single recommendation can hold across densities |

## 9.5 Runs

Run length: 5 minutes, so that a vehicle changing every 30 s makes about 10 changes and chains of several changes are possible.

Pilot: two policies and the baseline, 10 seeds each, at 4 vehicles. It checks the pipeline and measures the spread of the primary measures over seeds.

Seeds: chosen from the pilot so that the 95 % confidence interval of tracking success is at most ± 0.1 wide. 10 per cell is the starting estimate.

Stage A, screening: all 15 policies and the baseline at 4 vehicles with spread speeds: 160 runs, about 15 hours one after another.

Stage B, conditions: the policies on or near the stage A frontier (at most 5) and the baseline, under all 6 combinations of vehicles and speeds: up to 360 runs, about 33 hours.

Runner: the batch runner of the adversarial agents design: one scenario over a seed list with overrides, one results directory per run.

## 9.6 Analysis

Unit: the run. Links within a run are not independent, so confidence intervals come from resampling runs (bootstrap over seeds), not links.

Frontier: a policy is dominated if another policy is better on both primary measures and the difference on at least one exceeds its confidence interval. The frontier is reported per condition.

Hard limit first: a policy that loses warnings beyond its silent time is reported, but excluded from the frontier.

Sensitivity: the frontier is recomputed with the staleness threshold at 0.5 s and 2 s, and with each attacker. A statement that does not survive these is reported as conditional on the setting it depends on.

Hypotheses: each is reported as supported, not supported or undecided, with the numbers.

## 9.7 Result

Charts: per condition, stale share across and tracking success up, one point per policy with its confidence intervals, the frontier drawn through the undominated points, and the chance level as a line.

Table: the frontier policies per condition, with both primary measures and their intervals.

Statements: of the form "with 4 vehicles at spread speeds, policy P gave tracking success T ± t at stale share S ± s; no tested policy was better on both", each followed by the limits of Section 12.

Test summary: a document in `results/` in the form of the existing summaries, and the raw per-run results kept for re-analysis.

## 9.8 What it can and cannot claim

The result is a comparison of countermeasures within the simulated conditions. It supports statements of the form "policy P is better than policy Q on both privacy and safety in this setup" and "this is what silence costs per unit of privacy here". It is the basis for choosing what to examine further.

It is not yet a recommendation to vehicle manufacturers. That needs evidence this harness cannot produce:

| Missing | Why it matters | Further step |
| --- | --- | --- |
| Radio channel | packet loss with distance and congestion adds to the gaps and changes the cost of silence | carry the policies into Artery (Vanetza with OMNeT++ and SUMO), which models the channel |
| Realistic traffic | privacy and receiver load depend on density, road networks and trips of hours | SUMO traffic on real road networks, at hundreds of vehicles |
| Attacker coverage | real tracking uses receivers at many places and over long periods | an attacker with receivers at a chosen share of intersections, scored over whole trips |
| Safety applications | stale information is not the same as a warning that came too late | a reference application (e.g. forward collision or intersection warning) and lost warning lead time |
| OBU hardware | CPU times on a VM do not carry over | per-operation times measured on an OBU or its security module |
| Validation | the setup has not been shown to reproduce known results | reproduce one or two published mix-zone or silent-period results; independent review |

# 10. Components to build

| Component | Proposed place | What |
| --- | --- | --- |
| Position recorder | `sim/images/mobility/` or the harness | every published position of every vehicle, with time, kept with the run's results |
| Disruption analysis | `sim/vnapsim/overhead.py` | reads a results directory (`messages.jsonl`, positions, station logs) and writes `overhead.json` |
| Verification collector | harness | records `vanetza/out/cam` with security reports at each receiver |
| CPU sampler | harness | cgroup CPU time of each station container, about 10 times a second |
| Station counters and timing | `patches/vanetza-nap/` | change timing line; counts and sized of signatures, verifications, chain checks, cache misses |
| Warning probe | scenario format and harness | DENMs at random times; delivery per receiver |
| Sweep scenarios | `sim/scenarios/` | baseline and variants of Section 4 |
| Metrics and report | `sim/vnapsim/check.py`, `service/vnapapi/report.py` | Section 7 |
| Batch runner | `sim/arena/`, shared with the adversarial agents design | one scenario over a seed list with overrides |
| Guessing attacker | `sim/images/eavesdropper/linkers/` | links each new pseudonym to a random silent track |
| Joint analysis | `sim/vnapsim/frontier.py` | bootstrap intervals, frontier per condition, sensitivity, charts and the result table |
| 8-vehicle scenario | `sim/scenarios/` | the crossing with 8 vehicles |

# 11. Phases

| Phase | Content | Done when |
| --- | --- | --- |
| 1 | position recorder; disruption analysis from `messages.jsonl` (gap, position error, stale time, ghost time) | on a run without a silent period, every gap is at most two CAM intervals; with a silent period, gaps match the logged silent periods |
| 2 | verification collector; warning probe | probe warnings during silent periods are reported lost or delayed, the others delivered |
| 3 | CPU sampler; baseline and sweep on the two scenarios | cost-per-change slopes with their spread over seeds |
| 4 | station counters and timing (new image tag) | counts per change and refill in `overhead.json` |
| 5 | metrics, report section, Results tab; a test summary in `results/` | the sweep reported in the form of the existing summaries |
| 6 | joint experiment: guessing attacker, joint analysis, pilot | the seed count per cell is settled |
| 7 | joint experiment: stages A and B, analysis, test summary | the frontier per condition and the hypotheses reported, with intervals and sensitivity |

Phases 1 to 3 need no new station image. Phases 6 and 7 can start after phase 3 with the measures that need no new image; the counts of phase 4 are then added by re-analysis where the data allows, or by rerunning.

# 12. Limits

| Limit | Effect | How to read the results |
| --- | --- | --- |
| Lossless link | no packet loss, congestion or range; one reference receiver hears what all hear | measured disruption is the part caused by the change, a lower bound of what a road would show |
| Shared CPUs | times depend on the host and the other containers | compare variants on one host; carry over through the operation counts |
| Few vehicles | receiver load from simultaneous changes grows with traffic density | include a run with 6 to 10 vehicles once the measures work |
| Neighbour table model | socktap keeps no table like a safety application would | ghost time is computed under a stated timeout, not observed |
| Position belief model | receivers do not estimate positions themselves | the two variants (last known, extended) bracket what an application might do |

# 13. Open questions

The staleness threshold: 1 s as proposed, or per application (collision warnings may need much fresher information).

Whether warnings should be exempt from silent periods, or end them. This is a change to the station behaviour and a trade between privacy and safety, to be measured before it is decided.

Which target hardware to use for carrying the operation counts over, and where its per-operation times come from.

The neighbour-table timeout to assume for ghost time.

Whether the sweep should also vary the CAM rate, since the minimum gap is two CAM intervals.

The choice of primary measures for the joint experiment (Section 9.3). They should be agreed before the pilot, because changing them afterwards weakens the result.

Whether several run instances can run in parallel on the host; stages A and B take about two days one after another.

Whether the host carries 8 vehicles with a PKI, an eavesdropper and the collectors.

# LET’S IMPLEMENT THIS

## My own (based on the reasoning and knowledge that I have)

| Group / Class |  | Measure / Metric |  | How it is measured / Logic |
| --- | --- | --- | --- | --- |
| Privacy | Tracking success ratio |  |  | How often the attacker /eavesdropper can link the identity of the vehicle, upon the change of pseudonyms |
|  | Mean / Maximum tracking time and distance |  |  | For how long (time / distance) an attacker can track a vehicle upon the modification of pseudonyms |
|  | Pseudonym lifetime / change frequency |  |  | For how long each identifier is kept in use. |
| PKI / privacy | Pseudonym pool consumption | Certificate usage per time period versus the batch size, and how often the vehicle needs to reconnect to RA to request more certs. |
|  | Authority overhead | Provisioning latency, number of certificates provided and storage requirements on the vehicle. |
|  | Revocation cost | CRL size, and for how long does the revocation misbehaviour takes effect. |
| Safety | Packet loss and awareness | Delivery ratio of CAM messages, inter packet gap (does the pseudonym change introduces impact on delivery of such messages ?) Includes metrics from section 6.1 and 14.4 |
|  | Silent-period duration | Number of lost warnings, time-to-collision (TTC) violations, collision risk |
|  | Neighbour-table / cooperative-perception disruption | How many tracking IDs are lost per change and how much position-tracking error (CPM/CAM) the receivers has. |
| Security |  | Cryptographic overhead |  | Signing and verification latency, what are the message size. Includes the metrics in section 2.2 and 14.2 Includes the metrics in section 5.1 and 14.3 |
|  |  | Misbehaviour detection |  | When identities change what is the impact on the misbehaviour detection rate |
| Cost | Change synchronization ratio | How many neighbours change together (cooperatively or synchronously) |

Keep the metrics in sections 2.2 or  14.2 (as highlighted per Claude)

## 2.2 Already measured

| Measure | Where | Covers |
| --- | --- | --- |
| `<station>.pki.refresh_ms_mean`, `refresh_ms_max` | `vnapctl check` | time from a refill request to its installation |
| `<station>.pki.derivation_ms_mean` | `vnapctl check` | key derivation per batch |
| `<station>.pki.starved` | `vnapctl check` | changes refused for lack of certificates |
| `pki.issue_ms_max`, `pki.queue_ms_max` | `vnapctl check` | work and queueing at the PKI |
| `silent period over, <n> frame(s) suppressed` | station log | frames dropped per silent period |
| `silent.share` | summary report facts | share of the vehicles' time spent silent |
| `rsu.cam.rx_per_s` | `vnapctl check` | CAMs per second at the RSU, over the whole run |
| every frame on the link, with time, sender identifiers and position | eavesdropper `messages.jsonl` | raw material for the disruption measures (Section 6) |

## 5.1 On the OBU

| Work | When | How to measure |
| --- | --- | --- |
| ID change handshake and switch | every change | a timing log line at COMMIT, e.g. `[IDCHANGE] completed in <us> us` (new, station code) |
| signing | every message; unchanged by a change | counter of signatures made (new) |
| key derivation and key check | every refill, per certificate | exists: `derivation_ms_mean`; add a count and key size |
| chain check of new certificates | every refill, per certificate | counter and total time (new) and certificate size |
| all of the above, and anything missed | the whole run | CPU time of the station's container, sampled from its cgroup about 10 times a second (new, harness) |

## 6.1 Measures

| Measure | Definition | Why it matters |
| --- | --- | --- |
| Reception gap | per change and receiver: time from the last message under the old identity to the first under the new one | for that long the vehicle is missing from the receiver's view |
| Position error | the receiver's belief about the vehicle's position against its true position, over time; two variants: last known position, and last position extended by speed and heading | turns a gap into metres of uncertainty, which is what a collision warning depends on |
| Time stale | share of time the receiver's latest information about the vehicle is older than a threshold (proposed 1 s, the longest CAM interval of EN 302 637-2) | one number per vehicle and run, comparable across variants |
| Verification failures | received messages whose security report is not `Success`, within 5 s after a change | a receiver that cannot yet check the new certificate may discard messages |
| Ghost time | time the old identity remains in the receiver's neighbour table while the new one is already there, under a stated table timeout | the receiver briefly counts two vehicles where there is one, and loses the vehicle's history |
| Warning delay and loss | delay from triggering a DENM to its reception, and the share never received (Section 6.3) | the most direct safety cost: a silent period also drops warnings |

# 7. Outputs

Proposed `vnapctl check` metrics, so that a scenario can state limits:

| Metric (proposed) | Meaning |
| --- | --- |
| `<station>.change.ms_mean`, `.ms_max` | time of the change handshake on the station |
| `<station>.change.cpu_ms_mean` | extra CPU per change against the baseline window |
| `<station>.verify.chain_checks`, `.cache_misses` | work caused by other vehicles' changes |
| `<receiver>.gap_ms_max`, `.gap_ms_mean` | reception gaps around changes, over all senders |
| `<receiver>.stale_share` | share of time information about some sender was older than the threshold |
| `<receiver>.pos_error_m_p95` | 95th percentile of the position error during gaps |
| `denm.delay_ms_max`, `denm.lost` | warning probe results |